Table S1. Database of ignition time of wood.

| References | Wood species | heat flux (kW/m2) | moisture content (%) | density (kg/m3) | ignition time (s) |
| --- | --- | --- | --- | --- | --- |
| Chen et al. (2014) [1] | Paulownia | 12/17.8/30/39 | 15 | 280 | 160/50/16/7 |
| Seo et al. (2016) [2] | Japanese cedar/Korean fir/Japanese larch/red pine/giant dogwood/Manchurian ash | 50 | n/a | 310/320/460/540/590/700 | 9/19/18/24/33/40 |
| Grexa et al. (1997) [3] | Spruce | 30/50/65 | 10.2 | 506 | 62/18/10 |
| Grexa et al. (1997) [3] | Polar | 30/50/65 | 9.8 | 336 | 49/11/5.5 |
| Grexa et al. (1997) [3] | Oak | 30/50/65 | 10.4 | 620 | 110/28/15 |
| Grexa et al. (1997) [3] | Beech | 30/50/65 | 10 | 684 | 91/26/16 |
| Martinka et al. (2013) [4] | Norway spruce | 30/40 | 6 | 340 | 30/15 |
| Rocha and Landesmann (2016) [5] | Boleira/ Cabreuva/Cera cozida/Cedrilho/ Guapuruvu/Guavirova/Imbuia/Pinus caribaea/Pinus hondurensis | 50 | 0 | 352/675/750/445/346/780/610/346/490 | 21/57/61/25/16/93/45/21/48 |
| Hao et al. (2020) [6] | Oak | 20/35/50 | 0 | 724 | 1440/90/45 |
| Hao et al. (2020) [6] | Larch | 20/35/50 | 0 | 664 | 1215/65/25 |
| Hao et al. (2020) [6] | Red cedar | 20/35/50 | 0 | 353 | 725/50/20 |
| Öhrn et al. (2023) [7,8] | Spruce | 20/35/50 | 7 | 380 | 80/19/10 |
| Liu et al. (2023) [9] | Cypress | 25/35/50 | 0 | 772 | 121/43/16 |
| Liu et al. (2023) [9] | Fir | 25/35/50 | 0 | 385 | 68/29/12 |
| Hedayati et al. (2017) [10] | Southern yellow pine | 20/30/50 | 5 | 538 | 192/34/24 |
| Hedayati et al. (2017) [10] | Southern yellow pine | 20/30/50 | 10 | 550 | 357/71/25 |
| Hedayati et al. (2017) [10] | Southern yellow pine | 20/30/50 | 15 | 626 | 703/65/31 |
| Hedayati et al. (2017) [10] | Spruce-pine-fir | 20/30/50 | 5 | 392 | 233/32/10 |
| Hedayati et al. (2017) [10] | Spruce-pine-fir | 20/30/50 | 10 | 413 | 307/56/15 |
| Hedayati et al. (2017) [10] | Spruce-pine-fir | 20/30/50 | 15 | 423 | 465/49/17 |
| Nguyen et al. (2019) [11] | Poplar | 50 | 0 | 530 | 24 |
| Nguyen et al. (2019) [11] | Radiata pine | 50 | 0 | 410 | 28 |
| Batiot et al. (2014) [12] | Fir | 15/20/30/40/50/60 | 0 | 395 | 725/111/25/17/9/6 |
| Batiot et al. (2014) [12] | Fir | 15/20/30/40/50/60 | 9 | 438 | 1074/355/41/27/18/10 |
| Jin and Chung (2020) [13] | Cypress | 50 | 7.3 | 418 | 7 |
| Xu et al. (2015) [14] | Douglas fir | 25/50/75 | 14 | 470 | 169/23/7 |
| Xu et al. (2015) [14] | Scots pine | 25/50/75 | 17.5 | 460 | 206/16/6 |
| Xu et al. (2015) [14] | Southern pine | 25/50/75 | 17 | 420 | 225/29/8 |
| Xu et al. (2015) [14] | Shorea | 25/50/75 | 15.2 | 420 | 120/13/5 |
| Xu et al. (2015) [14] | Merbau | 50/75 | 19.5 | 860 | 148/29 |
| Mindykowski et al. (2019) [15] | Nordic spruce | 12.5/15/20/25/30/35 | 12 | 517 | 1588/804/217/89/53/39 |
| Mindykowski et al. (2019) [15] | Nordic spruce | 12.5/15/20/25/30/35 | 0 | 437 | 995/618/135/57/41/22 |
| Haurie et al. (2019) [16] | Tabebuia donnell-smithii | 35 | 8.9 | 448 | 37 |
| Haurie et al. (2019) [16] | Enterolobium cyclocarpum | 35 | 8.9 | 504 | 35 |
| Haurie et al. (2019) [16] | Tabebuia rosea | 35 | 9.0 | 604 | 56 |
| Haurie et al. (2019) [16] | Swietenia humilis | 35 | 9.9 | 655 | 68 |
| Haurie et al. (2019) [16] | Lysiloma acapulcensis | 35 | 9.2 | 685 | 72 |
| Haurie et al. (2019) [16] | Cordia elaeagnoides | 35 | 7.3 | 1130 | 93 |
| Haurie et al. (2019) [16] | Tabebuia chrysantha | 35 | 8.3 | 1234 | 144 |
| Martinka et al. (2017) [17] | Poplar wood | 30/40/50 | 0 | 452 | 35/17/10 |
| Moghtaderi et al. (1997) [18] | Pacific maple | 14/18/20/25/37/50/65 | 10 | 598 | 960/160/150/69/20/8/5 |
| Moghtaderi et al. (1997) [18] | Sugar pine | 20/25/35/45/55 | 10 | 473 | 150/49/13/6/3 |
| Moghtaderi et al. (1997) [18] | Radiata pine | 20/30/40/50/60 | 0 | 465 | 179/19/9/5/3 |
| Moghtaderi et al. (1997) [18] | Radiata pine | 20/30/40/50/60 | 15 | 535 | 295/52/18/11/7 |
| Moghtaderi et al. (1997) [18] | Radiata pine | 20/30/40/50/60 | 22 | 567 | 420/67/30/11/9 |
| Moghtaderi et al. (1997) [18] | Radiata pine | 20/30/40/50/60 | 30 | 605 | 540/93/36/19/11 |
| Hagen et al. (2009) [19] | Nordic spruce | 20/30/40/50/60 | 8.5 | 474 | 681/103/38/21/13 |
| Jaskółowski et al. (2014) [20] | Beech | 20/30/50 | 9.6 | 610 | 624/64/29 |
| Jaskółowski et al. (2014) [20] | Oak | 20/30/50 | 9.2 | 644 | 451/81/36 |
| Jaskółowski et al. (2014) [20] | Pine | 20/30/50 | 9.4 | 476 | 302/50/25 |
| Delichatsios et al. (2003) [21] | Australian radiata pine | 20/25/30/45/50 | n/a (~10) | 526 | 484/153/132/30/22 |
| Yuan et al. (2023) [22] | Cedar | 20/30/40 | 6.5 | 540 | 55/14/11 |
| Delichatsios (2005) [23] | Plywood | 25/35/50 | n/a (~10) | 550 | 112/49/30 |
| Grześkowiak (2017) [24] | Pine | 35 | 12 | 550 | 40 |
| Tran and White (1992) [25] | Redwood | 16/23/35/48 | 8.3 | 337 | 741/48/16/3 |
| Tran and White (1992) [25] | Southern pine | 15/23/34/51 | 9.7 | 557 | 909/85/31/5 |
| Tran and White (1992) [25] | Red oak | 16/23/34/50 | 8.5 | 716 | 930/118/50/13 |
| Tran and White (1992) [25] | Basswood | 15/23/34/48 | 8.1 | 454 | 182/60/16/5 |
| Harada (2001) [26] | Japan cedar | 25/30/40/50 | 3 | 299 | 43/38/12/9 |
| Harada (2001) [26] | Hiba abor-vitae | 25/30/40/50 | 3 | 422 | 75/42/18/12 |
| Harada (2001) [26] | Japanese red pine | 25/30/40/50 | 3 | 433 | 81/45/21/16 |
| Harada (2001) [26] | Japanese larch | 30/40/50 | 3 | 542 | 64/22/16 |
| Harada (2001) [26] | Japanese walnut | 25/30/40/50 | 3 | 547 | 185/55/23/17 |
| Harada (2001) [26] | Japanese beech | 25/30/40/50 | 3 | 581 | 155/59/31/22 |
| Harada (2001) [26] | Japanese oak | 30/40/50 | 3 | 772 | 113/54/27 |
| Harada (2001) [26] | Poulownia | 30/40/50 | 3 | 266 | 30/14/10 |
| Harada (2001) [26] | Zelkova | 30/40/50 | 3 | 703 | 92/36/22 |
| Spearpoint and Quintiere [27] | Redwood | 13 | 8.6 | 354 | 2170 |
| Spearpoint and Quintiere [27] | Douglas fir | 12 | 7.4 | 502 | 3633 |
| Spearpoint and Quintiere [27] | Maple | 12 | 4.8 | 741 | 3610 |
| Spearpoint and Quintiere [27] | Redwood | 9 | 7.4 | 328 | 1416 |
| Spearpoint and Quintiere [27] | Douglas fir | 9 | 8.5 | 455 | 2395 |
| Spearpoint and Quintiere [27] | Maple | 8 | 4.8 | 742 | 2680 |
| Shen et al. (2013) [28] | Aspen | 40 | 3.2/12.6/24.7/36 | 525/573/634/692 | 45/97/109/156 |
| Park and Lee (2008) [29] | Plywood | 50 | 10.3 | 540 | 35 |
| Lin et al. (2023) [30] | Merbau | 40/50 | 0 | 900 | 39/22 |
| Chung and Jin (2022) [31] | Spruce | 50 | 10.7 | 511 | 9 |
| Chung and Jin (2022) [31] | Pich pine | 50 | 10 | 419 | 17 |
| Chung and Jin (2022) [31] | Lauan | 50 | 8.8 | 543 | 10 |
| Chung and Jin (2022) [31] | Chest nut | 50 | 11 | 634 | 37 |
| Chung and Jin (2022) [31] | Oak | 50 | 12.1 | 976 | 51 |
| Chung and Jin (2022) [31] | Zelkova | 50 | 10 | 655 | 41 |

References

[1]	Chen X, Zhou Z, Li P, Zhou D, Wang J. Effects of sample orientation on pyrolysis and piloted ignition of wood. J Fire Sci 2014;32:483–97. https://doi.org/10.1177/0734904114534612.

[2]	Seo HJ, Park JE, Son DW. Combustion and thermal characteristics of Korean wood species. BioResources 2016;11:7537–50. https://doi.org/10.15376/biores.11.3.7537-7550.

[3]	Grexa O, Horvathova E, Osvald A. Cone Calorimeter Studies of Wood Species. Int Symp Fire Technol 1997:77–84.

[4]	Martinka J, Chrebet T, Král J, Balog K. An examination of the behaviour of thermally treated spruce wood under fire conditions. Wood Res 2013;58:599–606.

[5]	Rocha M, Landesmann A. Combustion properties of Brazilian natural wood species Monique. Fire Mater 2016;40:219–28. https://doi.org/10.1002/fam.2281.

[6]	Hao H, Chow CL, Lau D. Effect of heat flux on combustion of different wood species. Fuel 2020;278. https://doi.org/10.1016/j.fuel.2020.118325.

[7]	Öhrn O. Time to ignition for wood covered with ZnO. Luleå University of Technology, 2023.

[8]	Öhrn O, Sykam K, Gawusu S, Mensah RA, Försth M, Shanmugam V, et al. Surface coated ZnO powder as flame retardant for wood: A short communication. Sci Total Environ 2023;897:1–7. https://doi.org/10.1016/j.scitotenv.2023.165290.

[9]	Liu H, Li M, Jiang L, Xu Q. Experimental and theoretical study on ignition and combustion characteristics of aging woods by cone calorimetry. J Therm Anal Calorim 2023;148:10573–82. https://doi.org/10.1007/s10973-023-12311-0.

[10]	Hedayati F, Yang W, Zhou A. Effects of moisture content and heating condition on pyrolysis and combustion properties of structural fuels. Fire Mater 2018;42:741–9. https://doi.org/10.1002/fam.2528.

[11]	Nguyen TT, Nguyen TVK, Xiao Z, Wang F, Zheng Z, Che W, et al. Combustion behavior of poplar (Populus adenopoda Maxim.) and radiata pine (Pinus radiata Don.) treated with a combination of styrene-acrylic copolymer and sodium silicate. Eur J Wood Wood Prod 2019;77:439–52. https://doi.org/10.1007/s00107-019-01401-2.

[12]	Batiot B, Luche J, Rogaume T. Thermal and chemical analysis of flammability and combustibility of fir wood in cone calorimeter coupled to FTIR apparatus. Fire Mater 2014;38:418–31. https://doi.org/10.1002/fam.2192.

[13]	Jin E, Chung YJ. Fire risk assessment of cypress wood coated with metal oxide and metal silicate flame retardant using cone calorimeter. J Fire Sci 2020;38:504–21. https://doi.org/10.1177/0734904120948215.

[14]	Xu Q, Chen L, Harries KA, Zhang F, Liu Q, Feng J. Combustion and charring properties of five common constructional wood species from cone calorimeter tests. Constr Build Mater 2015;96:416–27. https://doi.org/10.1016/j.conbuildmat.2015.08.062.

[15]	Mindykowski P, Jørgensen M, Svensson S, Jomaas G. A simple correlation for monitoring the ignition propensity of wet nordic spruce wood. Fire Saf J 2019;107:186–92. https://doi.org/10.1016/j.firesaf.2018.08.001.

[16]	Haurie L, Giraldo MP, Lacasta AM, Montón J, Sonnier R. Influence of different parameters in the fire behaviour of seven hardwood species. Fire Saf J 2019;107:193–201. https://doi.org/10.1016/j.firesaf.2018.08.002.

[17]	Martinka J, Hroncová E, Kačíková D, Rantuch P, Balog K, Ladomerský J. Ignition parameters of poplar wood. Acta Fac Xylologiae Zvolen 2017;59:85–95. https://doi.org/10.17423/afx.2017.59.1.08.

[18]	Moghtaderi B, Novozhilov V, Fletcher DF, Kent JH. A New Correlation for Bench-scale Piloted Ignition Data of Wood. Fire Saf J 1997;29:41–59. https://doi.org/10.1016/S0379-7112(97)00004-0.

[19]	Hagen M, Hereid J, Delichatsios MA, Zhang J, Bakirtzis D. Flammability assessment of fire-retarded Nordic Spruce wood using thermogravimetric analyses and cone calorimetry. Fire Saf J 2009;44:1053–66. https://doi.org/10.1016/j.firesaf.2009.07.004.

[20]	Jaskółowski W, Ogrodnik P. The study of time to ignition of woods under external heat flux by piloted ignition and autoignition. For Wood Technol 2014;137:133–7.

[21]	Delichatsios M, Paroz B, Bhargava A. Flammability properties for charring materials. Fire Saf J 2003;38:219–28. https://doi.org/10.1016/S0379-7112(02)00080-2.

[22]	Yuan S, Sun X, Wang W, Zhou B, Sun X, Sun J, et al. The Reaction-to-fire Performance of Wood Covered with A Transparent Film: A Potential Method for the Preservation of Chinese Wooden Historical Buildings. Int J Archit Herit 2023;17:1778–90. https://doi.org/10.1080/15583058.2022.2070050.

[23]	Delichatsios MA. Piloted ignition times, critical heat fluxes and mass loss rates at reduced oxygen atmospheres. Fire Saf J 2005;40:197–212. https://doi.org/10.1016/j.firesaf.2004.11.005.

[24]	Grześkowiak W. Effectiveness of new wood fire retardants using a cone calorimeter. J Fire Sci 2017;35:565–76. https://doi.org/10.1177/0734904117737464.

[25]	Tran HC, White RH. Burning rate of solid wood measured in a heat release rate calorimeter. Fire Mater 1992;16:197–206. https://doi.org/10.1002/fam.810160406.

[26]	Harada T. Time to Ignition , Heat Release Rate and Fire Endurance Time of Wood in Cone Calorimeter Test. Fire Mater 2002;25:161–7. https://doi.org/10.1002/fam.766.

[27]	Spearpoint MJ, Quintiere JG. Predicting the piloted ignition of wood in the cone calorimeter using an integral model — effect of species, grain orientation and heat flux. Fire Saf J 2001;36:391–415. https://doi.org/doi.org/10.1016/S0379-7112(00)00055-2.

[28]	Shen D, Xiao R, Fang M, Chow W. Thermal-balanced integral model for pyrolysis and ignition of wood. Korean J Chem Eng 2013;30:228–34. https://doi.org/10.1007/s11814-012-0098-9.

[29]	Park J-S, Lee J-J. Ignition and Heat Release Rate of Wood-based Materials in Cone Calorimeter Tests. Mokchae Konghak 2008;36:1–8.

[30]	Lin S, Qin Y, Huang X, Gollner MJ. Use of Pre-Charred Surfaces to Improve Fire Performance of Wood. Fire Saf J 2023;136:103745. https://doi.org/10.1016/j.firesaf.2023.103745.

[31]	Jin YCE. Prediction of Fire Risk Ratings of Wood Species for Fire Prevention Design 2022;36:64–73.